# 3D Printing Summary

| Metrics | 3D Printed Braille |
| --- | --- |
| Total Print Time (min) | 140 |
| Total Number of Components | 19 |
| Typical Total Mass (g) | 18 |
| Typical Number of Print Setups | 19 |

# 3D Printing Settings:

Important Notes:

- Ensure all Braille is printed vertically (shown in picture below)

- Print each label individually to eliminate stringing or blobbing in print

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support (Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nemeth_zero.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_one.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_two.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_three.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_four.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_five.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_six.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_seven.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_eight.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_nine.stl | 1 | 0:07 | .5 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_division_sign.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_minus_sign.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_multiplication_sign.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_plus_sign.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_decimal_point.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_enter.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_ground.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_reset.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |
| Nemeth_start.stl | 1 | 0:07 | 1 | 20 | N | 0.2/0.4 | Print in orientation given in STL. |

# Post-Processing

- If you notice any blobs on the side of the Braille tiles try using a blade to clean up the prints. Braille is hyper specific so any bumps or abnormalities on the print that could affect readability should be removed.

# Customization Options

- Braille is standardized so any changes in the model could render the tiles unreadable. Therefore, do not:

  - Resize the Braille

  - Alter the model in the slicer

  - Change the Braille dot orientation in any way

# Examples of Quality Prints:

- Below are some photos of acceptable prints and one example of what issues can occur if you print all together

Good Example of Prints Using Above Settings:

Below is how the Braille should be printed:

Example of Stringing When Printed in Groups Rather Than 1 by One:

This can occur when printed in batches. May not occur on all printers and can be a relatively simple clean up. But to avoid this please print one Braille tile at a time